SOP for IllusionGamePhysio

- Preparation

  - Charge Muse and BITalino

  - Turn on thermometer

  - Put DO NOT DISTURB sign on lab door

  - Recording machine = SOUND OFF; experiment machine = SOUND ON

- Box

  - Add preliminary comments in “IllusionGamePhysio” folder

  - Add more comments throughout data collection session

- jsPsychMarkers Preparation

  - Ensure that BOTH the recording and experiment machines are connected to the SAME network connection (University ethernet)

  - Check IPv4 address is same: type “ipconfig” into Python terminal

  - Run lsl_bridge.py on recording machine

  - Run localhost:8002 port on experiment machine:

- Set up experiment machine (participant’s screen)

  - SOUND ON

  - Open IllusionGamePhysio repo on VScode

  - Fit the LUX photosensor to top-left of screen

  - Fill in PP ID and open Information Sheet and Consent Form for PP

- Set up signals

  - BITalino: RESP (A1), ECG (A2), LUX (A3)

  - Muse EEG headband

- Set up LabRecorder

  - Study root: IllusionGamePhysio

  - Participant: P00X

  - Block task: RS, TAP, HCT, CC, IG

- Experimental procedure

  - Information Sheet and Consent Form

  - Resting State

  - Interoception battery (tasks and questionnaires)

  - COUNTERBALANCE: cognitive control tasks (flanker, Simon) and Illusion Game

  - Debrief